Directions   Activity 5 of 8   |   Time: 25 min

# Suricata: writing a first detection rule

Name: ______________________   Partner: ______________________   Date: __________

## Goal

Write your own intrusion detection rule and watch it fire when a partner pings you.

## You need

- Your Kali Purple VM on the host-only lab network

- A partner or the target VM that can ping your Kali machine

Safety: Run Suricata only on the lab network interface.

## Steps

- Install Suricata if needed and download the free community rules.

sudo apt install suricata -y

sudo suricata-update

- Open the settings file. Set HOME_NET to "[192.168.56.0/24]", set the af-packet interface to your lab interface, and under rule-files: add a line for /etc/suricata/rules/local.rules.

sudo nano /etc/suricata/suricata.yaml

- Create your rule file with one rule that alerts on any ping to the lab network.

echo 'alert icmp any any -> $HOME_NET any (msg:"LAB ICMP ping detected"; sid:1000001; rev:1;)' | sudo tee /etc/suricata/rules/local.rules

- Test the settings, then run Suricata on the lab interface.

sudo suricata -T -c /etc/suricata/suricata.yaml -v

sudo suricata -c /etc/suricata/suricata.yaml -i eth0

- In a second terminal, watch for alerts while your partner pings your Kali VM.

sudo tail -f /var/log/suricata/fast.log

- Label the parts of your rule: action, protocol, source, direction, destination, then the options msg, sid, and rev.

- Stretch: have the target VM run nmap -sS against your Kali VM and see which community rules fire. Detailed JSON alerts, the kind a SIEM collects, are in /var/log/suricata/eve.json.

## Check your work

Copy one alert line from fast.log:

What would you change so the rule alerts on TCP traffic to port 80 instead?

What is the sid for?